Report

about

Quad-rotors and payload system dynamics

By:    Ran  ,    July  2017

Table of Contents

Introduction	2

Nomenclature	3

Single quadrotor dynamics	4

General	4

Analysis assumptions	4

The quadrotor parts and elements	4

motion equations per the Newton-Euler method	5

motion equations per the Lagrange method	5

Appendix A – system elements specific models	11

Propeller	11

General:	11

Blade characteristics:	11

Propeller characteristics:	11

Thrust of the propeller:	11

Moments of the propeller:	12

ESC	13

General	13

Characteristics:	13

Electric motor	14

General	14

Characteristics:	16

Introduction

In this paper, I will describe the dynamical system equations of 2 Quad-Rotor (aka quad) units, utilizing a common payload. The equations describe each part with its full 6 Degree-of-Freedom (orientation/attitude and position), and then implementing some assumptions to constrain the model for 2D plain.  The total number of the system’s D.O.F will be described as well.

First I’ll show the dynamics of a single quad. The equations will be derived using the Newton method, and the Lagrangian method. To show correspondence between the two methods.

Then the equations of the entire system will be derived using only Lagrangian method, because it is assumed to be easier in the mathematical sense.

Perhaps some limiting cases will be shown, to verify the model.

The after all purpose is to analyze the system stability according to ‘weakly-nonlinear’ method.

Representative diagram for the dynamic system is shown here:

Figure 1 - system views

Nomenclature

For aerodynamic propeller model :

σ	: solidity ratio

a	: 2D lift coefficient slope

μ	: advance ratio

λ	: inflow ratio

𝛎	: induced velocity

ρ 	: air density

Rrad	: rotor radius

θ0 	: pitch of incidence

θtw	: twist pitch

Cd	: drag coefficient at 70% radial station

For the system model :

l	: distance propeller axis to CoG

h 	: distance propeller plane to CoG

RBI  	: rotation matrix from Body to Inertial coordinate system

RIB  	: rotation matrix from Inertial to Body coordinate system

Acronyms

ESC –  electronic speed control

BEC -  battery eliminator circuit

PWM – pulse-width modulation

FOC - Field Oriented Control (related to brushless motors)

The system dynamics

The examined system is composed of 2 units of quadrotors, and 1 payload which is connected to each of the quadrotors. And by that it is connecting between the 2 quads.

The system will be described in the 3D world. But will be decreased to the 2D case in order to simplify the future analysis work.

The used assumptions for the system analysis are:

Quadrotor:

- Quad body and parts are rigid. No elasticity is considered.

- Geometry structure is symmetrical in relation to the principal axes. And the mass distribution is uniform. Hence the Inertia matrix is taken as pure diagonal.

Payload construction:

- The ‘cable’ which the payload is connected to is straight, with constant length, and has no mass – it is actually a massless rod.

- The rod is connected to the quadrotor exactly in its center of mass (C.G).

- No friction nor moments are present in the rod connection points.

- The payload is ‘lumped mass’ – a mass that is centered in 1 mathematical point.

Propeller and aerodynamics:

- Ground effect is not taken in account because the desired balance point for investigation is high above ground (usually G.E is considered when below  4Rrad away).

- Disturbance on the payload, as result of downwash is also not taken in account. Assuming the payload cable (or rod) is long enough.

Electric motors:

- The delay for the motors and the ESC to react for the controller commands – is not considered here. Assuming by that, that the treated dynamics is ‘slow enough’.

* those issues of course might need to be re-considered in later phase of the work. So the analysis will be more realistic.

Specific parts dynamics :

Dynamics of the single quadrotor is described in appendix __

Dynamics of the ‘stand-alone’ payload (which is similar to spherical pendulum) is described in appendix __

The over-all dynamic equations of the system :

System of 1 quad and 1 payload is considered by 2 methods: Newton-Euler equations , and Lagrange method equations.

The two outcomes will be compared, and should be identical.

The whole system dynamics will be described using the Lagrange method.

Coordinate systems , and Rotation matrices

I – inertial coordinates frame. It is the global reference point for the problem.

B – Body coordinate frame. Located at the center of mass (C.G) of the quad.

XB is directed to Motor #1. Coor.Sys B is rotated by Euler angles ψ, θ,ϕ , relative to frame I.

P – Payload coordinate frame. The origin is also located at the C.G of the quad.

XP is directed to the payload mass. Coor.Sys P is rotated by Euler-similar angles γ,β relative to frame B.

γ angle :positive from XB ,around ZB,

β angle :positive from (-ZB) towords the rod location.

Rotation matrix from Body to Inertial frame :

RBI=RψRθRϕ=  c(θ)c(ψ)c(ψ)s(θ)s(ϕ)-c(ϕ)s(ψ)c(ϕ)c(ψ)s(θ)+s(ϕ)s(ψ)c(θ)s(ψ)c(ϕ)c(ψ)+s(θ)s(ϕ)s(ψ)-c(ψ)s(ϕ)+c(ϕ)s(θ)s(ψ)-s(θ)c(θ)s(ϕ)c(θ)c(ϕ)

* Cos∙is noted as c∙,  Sin∙is noted as s(∙)

Appendix 1 - Single quadrotor dynamics

General

The quad-rotor has 4 body arms and 4 electrical motors – each produces thrust dependent on the rotation speed of the rotors blades. At nominal state, all the rotors are rotating at the same constant speed, with direction shown in Fig.1.  The engines are numbered 1-4 as shown in the figure.

2.

1.

3.

4.

Figure 1

Rotors 1 and 3 are both Clockwise,

Rotors 2 and 4 are both CounterClockwise.

The quadrotor parts and elements

The quad is assembled of few mechanical and electrical parts:

| 1 | Propeller (x4) | 2 are appropriate for CW direction rotation, and 2 for CCW rotation |
| --- | --- | --- |
| 2 | Electric motor (x4) | Brushless DC |
| 3 | ESC (electronic speed control) | controls the electric motors torque output, by input of a varying voltage |
| 4 | MicroController | the electric unit that controls all the ESCs |
| 5 | Battery | usually LiPo type, with several other varying properties i.e : maximum possible current, nominal voltage |
| 6 | IMU sensor (6 to 10 D.O.F) | measures the unit linear accelerations, and angular velocities. Might also measure barometric pressure. Might also include a Magnetometer |

Each of these parts have a certain behavior. Their output will be in accordance with some input.

The elements equations are listed below, and the more elaborated models are described in the Appendix XX.

motion equations per the Newton-Euler method

..

motion equations per the Lagrange method

..

I will consider the general coordinates of the quad:

⁠xIyIzI⁠ is the location in space. Relative to inertial coor. system.

⁠ϕθψ is the orientation relative to the inertial coor. system.

Hence the 6 D-O-F.

Description of the rotaion between vector in the body coor.system to description in the inretial frame-work:

I will use the ‘Tait–Bryan angles’ notion.

Rϕ=1000Cosϕ-Sinϕ0SinϕCosϕ , Rθ=Cosθ0Sinθ010-Sinθ0Cosθ , Rψ=Cosψ-Sinψ0SinψCosψ0001

Rϕ- rotation about XB axis in Clockwise direction (negative right hand direction)

Rθ- rotation about Y’ axis in Clockwise direction (negative right hand direction)

(Y’ is the intermidiate Y-axis of the ‘temporary’ coordinate system of euler angles)

Rψ- rotation about ZI axis in Clockwise direction (negative right hand direction)

and so the overall rotation matrix (by matrix multiplication) from Body to Inertial is :

RBI=RψRθRϕ=  Cos(θ)Cos(ψ)Cos(ψ)Sin(θ)Sin(ϕ)-Cos(ϕ)Sin(ψ)Cos(ϕ)Cos(ψ)Sin(θ)+Sin(ϕ)Sin(ψ)Cos(θ)Sin(ψ)Cos(ϕ)Cos(ψ)+Sin(θ)Sin(ϕ)Sin(ψ)-Cos(ψ)Sin(ϕ)+Cos(ϕ)Sin(θ)Sin(ψ)-Sin(θ)Cos(θ)Sin(ϕ)Cos(θ)Cos(ϕ)

( the other-way rotation matrix will be RIB=RBI-1=RBIT )

When given Body coordinates  ⁠xByBzB one can represent them in Inertial system by:

⁠xIyIzI=RBI ⁠xByBzB

Angular velocity vector of the quad is :

ω= ϕ. XB+θ .Y'+ψ. ZI

For describing that vector in Body-coor.system we need to describe it in the relevant axes :

ZI=Rψ Rθ Rϕ ⁠xByBzB 0,0,1 	= ⁠-sinθcosθsinϕcosθcosϕ(xB,yB,zB)

Y'=Rθ Rϕ ⁠xByBzB 0,1,0 	= ⁠0cosϕ-sinϕ(xB,yB,zB)

XB= ⁠100(xB,yB,zB)

Define   Rpqr  =  10-sin⁡(θ)0cos⁡(ϕ)cos⁡(θ)sin⁡(ϕ)0-sin⁡(ϕ)cos⁡(θ)cos⁡(ϕ)

Using the 1st vector definition above we get , in body coordinates system , the (relative) angular rate :

ω= pqr  = Rpqr   ϕθψ    =   ⁠ϕ.-ψ.sin⁡(θ)ψ.cos⁡(θ)sin⁡(ϕ)+θ.cos⁡(ϕ)ψ.cos⁡(θ)cos⁡(ϕ)-θ.sin⁡(ϕ)⁠⁠

another thing the is influenced by the rotated coor.system is Moment of Inertia.

The matrix I = Ixx000Iyy000Izz is in Body system.

The inertia in inertial system is :  J= Rpqr  TI Rpqr    (rotated about the axis of rotation).

3. Energies of the system :

In order to derive the equations of motion, I’ll use the Lagraunge method.

Therefor I’ll use the lagrangian which is :  L=T-V

Kinetic energy ( translational + rotational contributioins ) :

T=12m V.V=12mv2+12Ic.gω2

Potential energy:

V=-M g zI   (z positive is downwards)

Position of vector in Body system is : location of the C.G with addition of location in that system:

X= XI+XB=XI+RBI XB

- V=XI+∂∂tRBI XB

As long as the system is only a rigid body then v=XI , and ω2=p2+q2+r2 (the angular rates we showed above).

T=12mxo2+yo2+zo2+12(Ixx p2+Iyy q2+Izz r2)

(again – it would be more complicated argument if we considered a non symetric rigid body)

The Lagrange equation are:

∂∂t∂L∂qi-∂L∂qi=∂∂t∂T∂qi-∂T∂qi+∂V∂qi=Qi          ;      i=1÷6

∂∂t∂L∂xo=m xo   	,   ∂L∂xo=0→ xo is 'ignorable' (cyclic)

∂∂t∂L∂yo=m yo   	,   ∂L∂yo=0→ yo is 'ignorable' (cyclic)

∂∂t∂L∂zo=m zo   	,   ∂L∂zo=m g zI

∂∂t∂L∂ϕ-∂L∂ϕ=-Ixx sin⁡(θ)ψ¨-θ.Ixxψ.cos⁡(θ)+Ixxϕ¨+θ.ψ.(-Iyycos⁡(θ)cos2(ϕ)+Iyycos⁡(θ)sin2(ϕ)+Izzcos⁡(θ)cos2(ϕ)-Izzcos⁡(θ)sin2(ϕ))+ψ.2(Izzcos2(θ)sin⁡(ϕ)cos⁡(ϕ)-Iyycos2(θ)sin⁡(ϕ)cos⁡(ϕ))+θ.2(Iyysin⁡(ϕ)cos⁡(ϕ)-Izzsin⁡(ϕ)cos⁡(ϕ))

∂∂t∂L∂θ-∂L∂θ=ψ.2(-Ixxsin⁡(θ)cos⁡(θ)+Iyysin⁡(θ)cos⁡(θ)sin2(ϕ)+Izzsin⁡(θ)cos⁡(θ)cos2(ϕ))+ϕ.(ψ.(Ixxcos⁡(θ)+Iyycos⁡(θ)cos2(ϕ)-Iyycos⁡(θ)sin2(ϕ)-Izzcos⁡(θ)cos2(ϕ)+Izzcos⁡(θ)sin2(ϕ))+θ.(Izzsin⁡(ϕ)cos⁡(ϕ)-Iyysin⁡(ϕ)cos⁡(ϕ)))+ψ¨(Iyycos⁡(θ)sin⁡(ϕ)cos⁡(ϕ)-Izzcos⁡(θ)sin⁡(ϕ)cos⁡(ϕ))+θ.ψ.(Iyysin⁡(θ)sin⁡(ϕ)cos⁡(ϕ)-Izzsin⁡(θ)sin⁡(ϕ)cos⁡(ϕ))+θ.ψ.(Izzsin⁡(θ)sin⁡(ϕ)cos⁡(ϕ)-Iyysin⁡(θ)sin⁡(ϕ)cos⁡(ϕ))+θ¨(Iyycos2(ϕ)+Izzsin2(ϕ))+θ.ϕ.(Izzsin⁡(ϕ)cos⁡(ϕ)-Iyysin⁡(ϕ)cos⁡(ϕ))

∂∂t∂L∂ψ-∂L∂ψ=-Ixxsin⁡(θ)ϕ¨+ψ¨(Ixxsin2(θ)+Iyycos2(θ)sin2(ϕ)+Izzcos2(θ)cos2(ϕ))+2θ.ψ.(Ixxsin⁡(θ)cos⁡(θ)-Iyysin⁡(θ)cos⁡(θ)sin2(ϕ)-Izzsin⁡(θ)cos⁡(θ)cos2(ϕ))+ϕ.(θ.(-Ixxcos⁡(θ)+Iyycos⁡(θ)cos2(ϕ)+Izzcos⁡(θ)sin2(ϕ))+ψ.(2Iyy cos2(θ)sin⁡(ϕ)cos⁡(ϕ)-2Izzcos2(θ)sin⁡(ϕ)cos⁡(ϕ)))+θ¨(Iyy cos⁡(θ)sin⁡(ϕ)cos⁡(ϕ)-Izzcos⁡(θ)sin⁡(ϕ)cos⁡(ϕ))+θ.ϕ.(-Iyy cos⁡(θ)sin2(ϕ)-Izz cos⁡(θ)cos2(ϕ))+θ.2(Izzsin⁡(θ)sin⁡(ϕ)cos⁡(ϕ)-Iyysin⁡(θ)sin⁡(ϕ)cos⁡(ϕ))

Some of the used trigonometric identities are:

cos2a=cos2a-sin2a=2cos2a-1

sin2a=2sinacosa

the general forces are :

Qx=i=14Rmxil-i=14Hxi+i=14Ti ZB∙ XI

Rmxil= CX AC ρ XX

ZB∙ XI=  Sin(ϕ)Sin(ψ)+Cos(ϕ)Sin(θ)Cos(ψ)

ZB∙ YI=-Sin(ϕ)Cos(ψ)+Cos(ϕ)Sin(θ)Sin(ψ)

ZB∙ ZI=  Cos(θ)Cos(ϕ)

Qy=i=14Rmyil-i=14Hyi+i=14Ti ZB∙ YI

Rmyil= CY AC ρ YY

QZ=DBody+Ddownwash+i=14Ti ZB∙ ZI

DBody – drag of the body frame and

Ddownwash - drag on the body frame parts, that are below the rotors and affected by the rotors downwash.

Summary

In this paper, I showed..

References

- S. BOUABDALLAH , “design and control of quadrotors with application to autonomous flying”, THÈSE NO 3727 (2007), Lausanne, EPFL

- Dynamics Modeling and Control of a Quadrotor with Swing Load

- flyingmachinearena Publications - http://flyingmachinearena.org/research/

- TI tutorial : https://training.ti.com/webinar-how-extend-flight-time-and-battery-life-quadcopters-and-industrial-drones

- Online course : https://www.coursera.org/learn/robotics-flight/

Appendix A – system elements specific models

Propeller

General:

The propeller has several blades. Each blade has certain geometry. When the blade is in some surrounding air velocity (certain speed and angle of attack) it has induced lift force and drag force.

Propeller is also named ‘rotor’.

The propeller is rotating at certain RPM speed. When it rotates, it creates: Thrust force, drag force, side force, drag moment, and rolling moment, relative to the hub (propeller center).

The propeller is considered to be with 2 blades.

An illustration for all those acting forces and moments:

…

Blade characteristics:

R0-start point of the blade , measured from hub center

R-end point of the blade , measured from hub center

θ0- initial twist angle of the blade

θtw- twist angle of the blade (constant linear twist)

Propeller characteristics:

n-number of blades for that propeller

Thrust of the propeller:

According to momentum theory and blade element theory (as in helicopters) – the forces and moments, on the propeller hub, created by the rotation of the propeller can be described as the followings:

- Thrust of the rotor acts perpendicular to the blades plane:

T=12 ρ A CT V2=CT A ρ ΩRrad2 =kT Ω2

CT= a σ(-λ4+(16+μ24)θ0-18(1+μ2)θtw)

kT=CT A ρ Rrad2

- Side force on each of the motors’ Hubs.  it is the outcome of the forces on the blades in their horizontal direction.

H=CH A ρ ΩRrad2  =kH Ω2

CH= a σ μ (14aCd_+14λ(θ0-θtw2))

kH=CH A ρ Rrad2

Moments of the propeller:

The resultant moments on the system are:

- Drag moment -  the result of moments by drag forces on the blades ( in their horizontal direction )

Q=CQA ρ ΩRrad2 Rrad=kQ Ω2

CQ= a σ ((1+μ2)Cd_8a+λ(-λ4+θ06-θtw8))

kQ=CQA ρ Rrad3

- Rolling moment - integration over the entire rotor of the lift of each section acting at a given radius

Rm=CRm A ρ ΩRrad2 Rrad =kR Ω2

CRm=-a μ σ(-λ8+θ06-θtw8)

kR=CRm A ρ Rrad3

ESC

General:

Electronic speed controller – the unit that is in charge of :

- translating the flight controller commands to the required voltage to the motor.

- Handling different input voltages, and output high current for the motor.

The result of this function is – enabling the controlling of variating rotation speed of the motor.

Schema example from TI manual for their ESC , illustrates that concept:

Figure 2 - ESC concept schem

The dynamics of the ESC is what matters to us. Because it inherence a delay from the given command by the controller to the actual current and voltage output to the motor.

In some cases in the literature this delay is ignored.

In my dynamic analysis I’ll start with a similar assumption.

For future bookmark-keeping I’ll describe what delay it can implement.

Characteristics:

Detailed controller description and behavior of an ESC is described in reference(4).

A simplified description of the ESC reaction time can be described similar to what we know about behavior of electrical capacitance. It can be described mathematically as:

Exponent..

Electric motor

General:

The considered electric motor is of type Brushless Motor DC. Here is a general description :

In a BLDC motor, the electromagnets do not move; instead, the permanent magnets

rotate and the armature remains static. This gets around the problem of how to transfer

current to a moving armature. In order to do this, the commutator assembly is replaced by an

intelligent electronic controller. The controller performs the same power-distribution found in

a brushed DC-motor, but using a solid-state circuit rather than a commutator. BLDC motors

have many advantages over DC motors. A few of these are:

• High dynamic response

• High efficiency

• Long operating life

• Noiseless operation

• Higher speed ranges

BLDC's main disadvantage is higher cost which arises from two issues. First, BLDC

motors require complex electronic speed controllers to run. Brushed DC-motors can be

regulated by a comparatively trivial variable resistor (potentiometer or rheostat), which is

inefficient but also satisfactory for cost-sensitive applications.

BLDC motor drives, systems in which a permanent magnet excited synchronous motor is fed

with a variable frequency inverter controlled by a shaft position sensor. There appears a lack

of commercial simulation packages for the design of controller for such BLDC motor drives.

One main reason has been that the high software development cost incurred is not justified

for their typical low cost fractional/integral kW application areas such as NC machine tools

and robot drives, even it could imply the possibility of demagnetizing the rotor magnets

during commissioning or tuning stages. Nevertheless, recursive prototyping of both the motor

and inverter may be involved in novel drive configurations for advance and specialized

applications, resulting in high developmental cost of the drive system. Improved magnet

material with high (B.H), product also helps push the BLDC motors market to tens of kW

application areas where commissioning errors become prohibitively costly. Modeling is

therefore essential and may offer potential cost savings.

A brushless dc motor is a dc motor turned inside out, so that the field is on the rotor

and the armature is on the stator. The brushless dc motor is actually a permanent magnet ac

motor whose torque-current characteristics mimic the dc motor. Instead of commutating the

armature current using brushes, electronic commutation is used. This eliminates the problems

associated with the brush and the commutator arrangement, for example, sparking and

wearing out of the commutator-brush arrangement, thereby, making a BLDC more rugged as

compared to a dc motor. Having the armature on the stator makes it easy to conduct heat

away from the windings, and if desired, having cooling arrangement for the armature

windings is much easier as compared to a dc motor.

Fig 2.1 Cross-section view of a brushless dc motor

Model equations from reference documents:

Characteristics: