ME-Project Report

about

Quad-rotors & Payload system dynamics and stability

By:    Ran  ,    Aug  2017

Table of Contents

Introduction	2

Nomenclature	3

The system dynamics	4

Coordinate systems , State variables, and Rotation matrices	4

Kinematics	6

Trimming to 2D dynamics	8

Summary – for initial part	11

References	12

Appendix 1 - Single quadrotor dynamics	13

General	13

motion equations per the Lagrange method	13

Appendix 2 – Spherical pendulum	16

General	16

Kinematics	16

Equations of motion , by Newton method	17

Equations of motion , by Lagrange method	17

Appendix A - rotation matrix detailed	18

Rotation matric from body to inertial frame	18

Rotation matrix from payload to body frame	18

Euler angular velocities , in Body frame	19

Inertia rotation	20

Introduction

Requirements:

required 2D  coordinates (inertial references frames),

connection between quads and payload (elastic)

the non-dimensional length and time scales,

the maneuver (hover , translation of payload from points A to B),

generalized forces (payload)

expected limiting cases.

ME-project contents

1 introduction

2 2D problem formulation

*general R^9 (2 rifig-body quads and a rigid-body payload) vs limiting cases

3 equilibrium analysis

*hover with wind force on payload vs specified motion

4 asymptotic analysis

*for selected limiting cases that reveal a Hopf bifurcation and/or an orbital instability

5 numerical analysis

*for asymptotic validation vs general maneuver

6 discussion

7 closing

In this paper, I will describe the dynamical system of 2 Quad-Rotor (aka quad) units, utilizing a common payload.

The problem formulation assumes 2D framework. The original 3D case is not treated here.

The quads full dynamics and control is not described here, and it is treated as given. Meaning the quads are inputs for the dynamics of the payload, and that is what I intend to investigate here.

The dynamic equations of the quads and payload will be described, and some limiting cases will be shown to verify the model.

The next step in this work will be to analyze the equations by Multiple Scales method or Averaging method.

A representative diagram for the system is shown here:

Figure 1 - system view

Nomenclature

l	: distance propeller axis to CoG

h 	: distance propeller plane to CoG

RBI  	: rotation matrix from Body to Inertial coordinate system

RIB  	: rotation matrix from Inertial to Body coordinate system

Acronyms

.

The system dynamics

The examined system is composed of 2 units of quadrotors, and 1 payload which is connected to each of the quadrotors. And by that it is connecting between the 2 quads.

The system is described in the 2D world.

The used assumptions for the system analysis are:

Quadrotor:

- Quad body and parts are rigid. No elasticity is considered.

- Geometry structure is symmetrical in relation to the principal axes. And the mass distribution is uniform. Hence the Inertia matrix is taken as pure diagonal.

Payload construction:

- The ‘cable’ which the payload is connected to is modeled as straight spring, with initial length l0, and has no mass.

- The cable is connected to the quadrotor exactly in its center of mass (C.G).

- No friction nor moments are present in the spring connection points.

- The payload is a rectangular box, characterized with Ip as it’s inertia metrix.

The over-all dynamic equations of the system :

The system of 2 quads and 1 mass payload is described using the Lagrange method.

Coordinate systems , State variables, and Rotation matrices

I – inertial coordinates frame. It is the global reference point for the problem.

It’s axes are : ⁠xI,yI,zIT

Bi – Body i coordinate frame. Located at the center of mass (C.G) of the quad.

XB is directed to Motor #1. Coor.Sys Bi is rotated by Euler angles ψ, θ,ϕ , relative to frame I.

Position of Bi relative to frame I : ⁠xi,yi,ziT , (i=1,2 - to address quad 1 and quad 2).

P – Payload coordinate frame. The origin is also located at the C.G of the quad.

XP is directed to the payload mass. Coor.Sys P is rotated by Euler-similar angles γi,βi relative to frame Bi (i=1,2).

γ angle :positive from XB ,around ZB,

β angle :positive from (-ZB) towords the rod location.

The used rotation matrices are developed in appendix A. the results summary are the followings:

- Rotation matrix from Body to Inertial frame :

RBI=RψRθRϕ=  c(θ)c(ψ)c(ψ)s(θ)s(ϕ)-c(ϕ)s(ψ)c(ϕ)c(ψ)s(θ)+s(ϕ)s(ψ)c(θ)s(ψ)c(ϕ)c(ψ)+s(θ)s(ϕ)s(ψ)-c(ψ)s(ϕ)+c(ϕ)s(θ)s(ψ)-s(θ)c(θ)s(ϕ)c(θ)c(ϕ)

* Cos∙is noted as c∙,  Sin∙is noted as s(∙)

- Euler angles rates vector is (ϕ. ,θ .,ψ.) but it’s not in Orthogonal coordinate framework.

To calculate it in a Body coor. sys. We can use the conversion matrix :

REulerPQR  =  10-sin⁡(θ)0cos⁡(ϕ)cos⁡(θ)sin⁡(ϕ)0-sin⁡(ϕ)cos⁡(θ)cos⁡(ϕ)

and ω= pqr  = REulerPQR   ϕθψ

- Rotation matrix from Payload to Body framework :

RPB=CβCγ-SγCγSβCβSγCγSβSγ-Sβ0Cβ

Kinematics

Locations of the system elements in inertial frame :

X1=x1y1z1φ1θ1ψ1       ,      X2=x2y2z2φ2θ2ψ2

Xp=x1ty1tz1t+l1 sinβtcosγtsinβtsinγt-cosβt      ;        0°<β<180°-180°<γ<180°

- Each quad position and orientation has it’s own general coordinates

- The payload is dependent and related to the position of the 1st quad.

But the orientation is described relative to Inertial coor.sys. (independent of the quad orientation)

The linear and angular velocities of the elements are :

Vtr2i=xi't2+yi't2+zi't2       ;    i=1,2

mi Vrot2=ωC.GTIC.GωC.G= ⅈi1pi2+ⅈi2qi2+ⅈi3ri2    ;   i=1,2

Vp2  =  l12β't2+sin2βtγ't2+

2l1sinβtγ'tcosγty1't-sinγtx1't+β'tcosβtcosγtx1't+cosβtsinγty1't+sinβtz1't+

x1't2+y1't2+z1't2

The Lagrangian L is L=T-V , where :

T=12m1Vtr21+12m1 Vrot21+12mpVp2+12m2Vtr22+12m2 Vrot22

V=m1gz1+m2gz2+mpg(z1-l1cosβ)

The total general coordinates are:

q=x1y1z1ϕ1θ1ψ1x2y2z2ϕ2θ2ψ2βγ  ;   which sums to 14 D.O.

Lagrange equations , before adding non-conservative forces :

The general forces (and moments) are according to :

Fthrust=00Thrusti1+Thrusti2+Thrusti3+Thrusti4;    i=1,2

(2)      Q1=RBI1Fthrust1 XI

Q2=RBI1Fthrust1 YI

Q3=RBI1Fthrust1 ZI

Q4=L1Thrust14-Thrust12

Q5=L1Thrust13-Thrust11

Q6=MotorMoment11-MotorMoment12+MotorMoment13-MotorMoment14

Q7=RBI2Fthrust2 XI

Q8=RBI2Fthrust2 YI

Q9=RBI2Fthrust2 ZI

Q10=L2Thrust24-Thrust22

Q11=L2Thrust23-Thrust21

Q12=MotorMoment21-MotorMoment22+MotorMoment23-MotorMoment24

Q13=Q14=0

(note: neglecting the aerodynamic forces on the payload, allows to set this last equation to 0)

The system dynamics equations are the result of the last equations and setting:

(3)  		(1)==(2)

Trimming to 2D dynamics

The above (3) equations are full equations for the 3D system. to get 2D dynamics I will set:

(4)   		yi→0,   ϕi→0,   ψi→0,   γ→0  ;  i=1,2

And also those variables derivatives are set to zero.

The dynamic equations I got are:

l1mpβ't2sinβt-β''tcosβt-mp+m1x1''t0-mpg+l1β''tsinβt+β't2cosβt+z1''t-m1g+z1''t0-ⅈ12θ1''t0-m2x2''t0-m2g+z2''t0-ⅈ12θ2''t0l1mp-gsinβt-l1β''t-cosβtx1''t-sinβtz1''t0

While noting that RBI is trimmed to be y axis rotation matrix :

cosθt0sinθt010-sinθt0cosθt

And the generalized forces are minimized to :

Q1=RBI1Fthrust1 XI

Q2=0

Q3=RBI1Fthrust1 ZI

Q4=0

Q5=L1Thrust13-Thrust11

Q6=MotorMoment11+MotorMoment13

for each of the quads.

This system is now 7 D.O.F.

Summary – for initial part

I described the 3D dynamics of system of 2 quadrotors and 1 connected lumped mass payload.

I also minimized the dynamics to 2D dimensions. It is changed from 14 D.O.F to 7 D.O.F.

‘TODO’ :

I see that after minimize to 2D dimension the 2nd quad is not fully depended on the lumped mass. I need to verify and check how to described the dynamic equations so it will be similar to triple-pendulum dynamics – those systems should behave very similar.

References

- S. BOUABDALLAH , “design and control of quadrotors with application to autonomous flying”, THÈSE NO 3727 (2007), Lausanne, EPFL

- Dynamics Modeling and Control of a Quadrotor with Swing Load

- flyingmachinearena Publications - http://flyingmachinearena.org/research/

- TI tutorial : https://training.ti.com/webinar-how-extend-flight-time-and-battery-life-quadcopters-and-industrial-drones

- Online course : https://www.coursera.org/learn/robotics-flight/

Appendix 1 - Single quadrotor dynamics

General

(4) 		M=Iα+ω×Iω

Using the skew matrix concept as in:

, which allows : , we get :

L(F4-F2)L(F3-F1)M1-M2+M3-M4 =I pqr + 0-rqr0-p-qp0 I pqr

Where I= Ixx000Iyy000Izz  using the assumption that the quad frame and parts are all symmetrical related to the body axes.

(5)		  LF4-F2LF3-F1M1-M2+M3-M4 = IxxpIyyqIzzr + 0-rqr0-p-qp0 IxxpIyyqIzzr

=Ixxp+(-Iyy+Izz)rqIyyq+(Ixx-Izz)rpIzzr+(-Ixx+Iyy)pq

So the motion equations are (3) and (5).

Equating to the ‘lagrange’ method equation requires further usage of the relations of (2).

motion equations per the Lagrange method

The Lagrangian is defined by  :

(6)  		L=T-V

Kinetic energy ( translational + rotational contributions ) :

T=12m1 V.V=12m1v2+12ωTIc.gω=12mx12+y12+z12+12Ixx p2+Iyy q2+Izz r2

Potential energy:

V=m1 g zI

(again – it would be more complicated argument if we considered a non-symmetric rigid body)

The Lagrange equations are, using (6), and the knowledge that V is not dependent on qi for mechanical systems :

∂∂t∂L∂qi-∂L∂qi=∂∂t∂T∂qi-∂T∂qi+∂V∂qi=Qi          ;      i=1÷6

∂∂t∂L∂xo=m xo   	,   ∂L∂xo=0→ xo is 'ignorable' (cyclic)

∂∂t∂L∂yo=m yo   	,   ∂L∂yo=0→ yo is 'ignorable' (cyclic)

∂∂t∂L∂zo=m zo   	,   ∂L∂zo=m g zI

∂∂t∂L∂ϕ-∂L∂ϕ=-Ixx sin⁡(θ)ψ¨-θ.Ixxψ.cos⁡(θ)+Ixxϕ¨+θ.ψ.(-Iyycos⁡(θ)cos2(ϕ)+Iyycos⁡(θ)sin2(ϕ)+Izzcos⁡(θ)cos2(ϕ)-Izzcos⁡(θ)sin2(ϕ))+ψ.2(Izzcos2(θ)sin⁡(ϕ)cos⁡(ϕ)-Iyycos2(θ)sin⁡(ϕ)cos⁡(ϕ))+θ.2(Iyysin⁡(ϕ)cos⁡(ϕ)-Izzsin⁡(ϕ)cos⁡(ϕ))

∂∂t∂L∂θ-∂L∂θ=ψ.2(-Ixxsin⁡(θ)cos⁡(θ)+Iyysin⁡(θ)cos⁡(θ)sin2(ϕ)+Izzsin⁡(θ)cos⁡(θ)cos2(ϕ))+ϕ.(ψ.(Ixxcos⁡(θ)+Iyycos⁡(θ)cos2(ϕ)-Iyycos⁡(θ)sin2(ϕ)-Izzcos⁡(θ)cos2(ϕ)+Izzcos⁡(θ)sin2(ϕ))+θ.(Izzsin⁡(ϕ)cos⁡(ϕ)-Iyysin⁡(ϕ)cos⁡(ϕ)))+ψ¨(Iyycos⁡(θ)sin⁡(ϕ)cos⁡(ϕ)-Izzcos⁡(θ)sin⁡(ϕ)cos⁡(ϕ))+θ.ψ.(Iyysin⁡(θ)sin⁡(ϕ)cos⁡(ϕ)-Izzsin⁡(θ)sin⁡(ϕ)cos⁡(ϕ))+θ.ψ.(Izzsin⁡(θ)sin⁡(ϕ)cos⁡(ϕ)-Iyysin⁡(θ)sin⁡(ϕ)cos⁡(ϕ))+θ¨(Iyycos2(ϕ)+Izzsin2(ϕ))+θ.ϕ.(Izzsin⁡(ϕ)cos⁡(ϕ)-Iyysin⁡(ϕ)cos⁡(ϕ))

∂∂t∂L∂ψ-∂L∂ψ=-Ixxsin⁡(θ)ϕ¨+ψ¨(Ixxsin2(θ)+Iyycos2(θ)sin2(ϕ)+Izzcos2(θ)cos2(ϕ))+2θ.ψ.(Ixxsin⁡(θ)cos⁡(θ)-Iyysin⁡(θ)cos⁡(θ)sin2(ϕ)-Izzsin⁡(θ)cos⁡(θ)cos2(ϕ))+ϕ.(θ.(-Ixxcos⁡(θ)+Iyycos⁡(θ)cos2(ϕ)+Izzcos⁡(θ)sin2(ϕ))+ψ.(2Iyy cos2(θ)sin⁡(ϕ)cos⁡(ϕ)-2Izzcos2(θ)sin⁡(ϕ)cos⁡(ϕ)))+θ¨(Iyy cos⁡(θ)sin⁡(ϕ)cos⁡(ϕ)-Izzcos⁡(θ)sin⁡(ϕ)cos⁡(ϕ))+θ.ϕ.(-Iyy cos⁡(θ)sin2(ϕ)-Izz cos⁡(θ)cos2(ϕ))+θ.2(Izzsin⁡(θ)sin⁡(ϕ)cos⁡(ϕ)-Iyysin⁡(θ)sin⁡(ϕ)cos⁡(ϕ))

Some of the used trigonometric identities are:

cos2a=cos2a-sin2a=2cos2a-1       ;       sin2a=2sinacosa

the general ‘forces’ Qi are the non-conservative ones:

- Motor thrusts

- Motor moments

- Possible Aerodynamic forces and moments

Q1=RBI.FthrustXI

Q2=RBI.FthrustYI

Q3=RBI.FthrustZI

Q4=l1Thrust4-Thrust2

Q5=l1Thrust3-Thrust1

Q6=MotorMoment1-MotorMoment2+MotorMoment3-MotorMoment4

The detailed final dynamic equations are:

==

Q=Thrust1+Thrust2+Thrust3+Thrust4sinθtcosψtcosϕt+sinψtsinϕtThrust1+Thrust2+Thrust3+Thrust4sinθtsinψtcosϕt-cosψtsinϕtThrust1+Thrust2+Thrust3+Thrust4cosθtcosϕtl1Thrust4-Thrust2l1Thrust3-Thrust1MotorMoment1-MotorMoment2+MotorMoment3-MotorMoment4

Appendix 2 – Spherical pendulum

General

Spherical pendulum consists of a lumped mass that is free to rotate 3D. its movement can be described by 2 rotation angles.

..schematics..

For simplicity the base coor.sys will be noted as Body frame.

Kinematics

The base of the hanging point (pivot) is assume as horizontal and constant.

The element that holds the mass and connects it to the pivot – is a rod, with a neglectable mass.

Rotation angles, as mentioned earlier above, are :

1.  γ :positive from XB ,around ZB,

2.  β :positive from (-ZB) towords the rod location.

Rotation order is γ first and then  β.

The location of the payload in Body frame is :

Xp|B=l1⁠SinβCosγSinβCosγ-Cosβ⁠

But in inertial frame it is :

(5)   				Xp|I=xByBzB+RBIl1⁠SinβCosγSinβCosγ-Cosβ⁠

Assuming here xByBzB=00z1, and RBI=I.

Note: when β=0, and γ=0 then Xp=00-l1

(6) 			Xp|I=l1cβtcγtβ't-sβtsγtγ'tl1cβtsγtβ't+cγtsβtγ'tsβtl1β't

Equations of motion , by Newton method

**this section will be written later on **

Equations of motion , by Lagrange method

L==T-V

T==12mpXp.2

V=mpgz1-l1Cosβ1

From (6) using some basic trigonometric identities, we get:

(7) 		Xp|I2=⁠l12β't2+sin2βtγ't2⁠

Calculating the Lagrange equation terms:

| q | β | γ |
| --- | --- | --- |
| ∂T∂q | l12mpsin⁡(β⁡(t))cos⁡(β⁡(t))γ'(t)2 | 0 |
| ∂T∂q | l12mpβ'(t) | l12mpsin2(β⁡(t))γ'(t) |
| ∂V∂q | gl1mpsin⁡(β⁡(t)) | 0 |
| ∂∂t∂T∂q | l12mpβ''(t) | 2l12mpβ'(t)sin⁡(β⁡(t))cos⁡(β⁡(t))γ'(t)+l12mpsin2(β⁡(t))γ''(t) |
| Q | 0 | 0 |

The resultant equations are:

(8)  	 gl1mpsinβt+l12mpβ''t+l12mpsinβt-cosβtγ't2=0

(9) 	               2l12mpβ'tsinβtcosβtγ't+l12mpsin2βtγ''t=0

Appendix A - rotation matrix detailed

Rotation matric from body to inertial frame

Description of the rotation between vector in the body coor.system to its description in the inertial frame-work:

I will use the ‘Tait–Bryan angles’ notion.

Rϕ=1000Cosϕ-Sinϕ0SinϕCosϕ , Rθ=Cosθ0Sinθ010-Sinθ0Cosθ , Rψ=Cosψ-Sinψ0SinψCosψ0001

Rϕ- rotation about XB axis in Clockwise direction (negative right hand direction)

Rθ- rotation about Y’ axis in Clockwise direction (negative right hand direction)

(Y’ is the intermediate Y-axis of the ‘temporary’ coordinate system of Euler angles)

Rψ- rotation about ZI axis in Clockwise direction (negative right hand direction)

and so the overall rotation matrix (by matrix multiplication) from Body to Inertial is :

RBI=RψRθRϕ=  Cos(θ)Cos(ψ)Cos(ψ)Sin(θ)Sin(ϕ)-Cos(ϕ)Sin(ψ)Cos(ϕ)Cos(ψ)Sin(θ)+Sin(ϕ)Sin(ψ)Cos(θ)Sin(ψ)Cos(ϕ)Cos(ψ)+Sin(θ)Sin(ϕ)Sin(ψ)-Cos(ψ)Sin(ϕ)+Cos(ϕ)Sin(θ)Sin(ψ)-Sin(θ)Cos(θ)Sin(ϕ)Cos(θ)Cos(ϕ)

( the other-way rotation matrix will be RIB=RBI-1=RBIT )

When given Body coordinates  ⁠xByBzB one can represent them in Inertial system by:

⁠xIyIzI=RBI ⁠xByBzB

Rotation matrix from payload to body frame

Similar to the Euler angles calculation , while replacing θ with β, and ψ with γ :

Rβ=Cosβ0-Sinβ010Sinβ0Cosβ , Rγ=CosγSinγ0-SinγCosγ0001

We get the rotation matrix between the coordinate frames Body to Payload, as :

RBP=RβRγ=CβCγCβSγ-Sβ-SγCγ0CγSβSβSγCβ

So payload location conversion between coor.systems is :
⁠xPyPzP=⁠00-l1=RBP ⁠lBlBlB

The other-way-around is :

RPB=RBPT=CβCγ-SγCγSβCβSγCγSβSγ-Sβ0Cβ

Euler angular velocities , in Body frame

Angular velocity vector of the quad is :

ω= ϕ. XB+θ .Y'+ψ. ZI

For describing that vector in Body-coor.system we need to describe it in the relevant axes :

ZI=Rθ Rϕ ⁠xByBzB 0,0,1 = ⁠-sinθcosθsinϕcosθcosϕ(xB,yB,zB)

Y'=Rϕ ⁠xByBzB 0,1,0 	= ⁠0cosϕ-sinϕ(xB,yB,zB)

XB= ⁠100(xB,yB,zB)

Define   Rpqr  =  10-sin⁡(θ)0cos⁡(ϕ)cos⁡(θ)sin⁡(ϕ)0-sin⁡(ϕ)cos⁡(θ)cos⁡(ϕ)

Using the 1st vector definition above we get , in body coordinates system , the (relative) angular rate :

ω= pqr  = Rpqr   ϕθψ    =   ⁠ϕ.-ψ.sin⁡(θ)ψ.cos⁡(θ)sin⁡(ϕ)+θ.cos⁡(ϕ)ψ.cos⁡(θ)cos⁡(ϕ)-θ.sin⁡(ϕ)⁠⁠

Inertia rotation

another thing the is influenced by the rotated coor.system is the Moment of Inertia.

The matrix I = Ixx000Iyy000Izz is in Body system.

The inertia in inertial system is :  J= Rpqr  TI Rpqr    (rotated about the axis of rotation).